Cat 5e Shield Modular RJ45 Connector

3U Gold-Plated Contacts

Model: RJ45-5ES

Main Features:

- Durable design: ≥ 1250 mating cycles;

- Trident head ensures stable contact for rapid transmission;

- 3μin gold-plated contacts for optimized signal integrity.

| Connector Type |  |
| --- | --- |
| Category | Cat 5e |
| Type | 8P8C (Eight Position, Eight Contact) |
| Shielding | Shielded |
| Material |  |
| Housing Material | Polycarbonate |
| Shielding Material | Brass with nickel plating |
| Contact Material | Copper |
| Contact Surface Material | Gold Plating on Nickel Barrier |
| Gold Plating Thickness | 3U (3μin or 0.0762 µm) |
| Electrical Properties |  |
| Voltage/Current Rating | 125 VAC / 1.5 A |
| Insulation Resistance | ≥ 500 MΩ |
| Contact Resistance | ≤ 20 mΩ |
| Dielectric Withstand Voltage | 1500 V DC/AC Peak |
| Mechanical Properties |  |
| Durability | ≥ 1250 mating cycles |
| Wire Gauge | 0.85 mm ≤ Ø ≤ 1.1 mm |
| Environment |  |
| Operating Temperature | -40°C to 70°C |
| Operating Humidity | RH 10% to 90% (non-condensing) |
| Storage Temperature | -40°C to 80°C |
| Storage Humidity | RH 5% to 96% (non-condensing) |
| General |  |
| Dimension | 23x12x8 mm (LxWxH) |
| Net Weight | 2.2 g |
| Diagram |  |